# Design and Implementation of an Hotel System Network Design (Project #3)

## Project #3 Case Study and Requirements

As a part of your end year networking project, you are required to design and implement Vic Modern Hotel network. The hotel has three floors; in the first floor there three departments (Reception, store and Logistics), in the second floor there are three departments (Finance, HR and Sales/Marketing), while the third floor hosts the IT and Admin. Therefore, the following are part of the considerations during the design and implementation;

- There should be three routers connecting each floor (all placed in the server room in IT department).

- All routers should be connected to each other using serial DCE cable.

- The network between the routers should be 10.10.10.0/30,10.10.10.4/30 and 10.10.10.8/30.

- Each floor is expected to have one switch (placed in the respective floor).

- Each floor is expected to have WIFI networks connected to laptops and phones.

- Each department is expected to have a printer.

- Each department is expected to be in different VLAN with the following details;
1st Floor;
- Reception- VLAN 80, Network of 192.168.8.0/24
- Store- VLAN 70, Network of 192.168.7.0/24
- Logistics- VLAN 60, Network of 192.168.6.0/24
2nd Floor;
- Finance- VLAN 50, Network of 192.168.5.0/24
- HR- VLAN 40, Network of 192.168.4.0/24
- Sales- VLAN 30, Network of 192.168.3.0/24
3rd Floor;
- Admin- VLAN 20, Network of 192.168.2.0/24
- IT- VLAN 10, Network of 192.168.1.0/24

- Use OSPF as the routing protocol to advertise routes.

- All devices in the network are expected to obtain IP address dynamically with their respective router configured as the DHCP server.

- All the devices in the network are expected to communicate with each other.

- Configure SSH in all the routers for remote login.

- In IT department, add PC called Test-PC to port fa0/1 and use it to test remote login.

- Configure port security to IT-dept switch to allow only Test-PC to access port fa0/1 (use sticky method to obtain mac-address with violation mode of shutdown.)

## Technologies Implemented

- Creating a network topology using  Cisco Packet Tracer.

- Hierarchical Network Design.

- Connecting Networking devices with Correct cabling.

- Creating VLANs and assigning ports VLAN numbers.

- Subnetting and IP Addressing.

- Configuring Inter-VLAN Routing (Router on a stick).

- Configuring DHCP Server (Router as the DHCP Server).

- Configuring SSH for secure Remote access.

- Configuring switchport security or Port-Security on the switches.

- Configuring WLAN or wireless network (Cisco Access Point).

- Host Device Configurations.

- Test and Verifying Network Communication.